Map a location using slam:

- Launch files for robot:

  - ros2 launch turtlebot3_gazebo turtlebot3_house.launch.py

2) Launch Cartographer:

- ros2 launch turtlebot3_cartographer cartographer.launch.py use_sim_time:=True

3) Move around to map:

- ros2 run turtlebot3_teleop teleop_keyboard

4) Save map:

- ros2 run nav2_map_server map_saver_cli -f maps/house_mapping_pratice_partial

Navigate with a generated map:

- Start the world in a map:

- ros2 launch turtlebot3_gazebo turtlebot3_world.launch.py

2) Open Navigation with gazebo simulation and the map file;

- ros2 launch turtlebot3_navigation2 navigation2.launch.py use_sim_time:=True map:=maps/my_map.yaml

3) Provide a 2d pose estimation relative to robots position

4) Use resource spawner to place objects for dynamic obstacle avoidance

Navigation with the SLAM toolbox (for custom robot or turtlebot 3):

- Start robot simulation in a world:

  - ros2 launch turtlebot3_gazebo turtlebot3_world.launch.py

- Start the SLAM navigation stack for nav2:

  - ros2 launch nav2_bringup navigation_launch.py use_sim_time:=True

- Start SLAM to generate map:

  - ros2 launch slam_toolbox online_async_launch.py use_sim_time:=True

- Start rviz to visualize everything:

  - rviz2

- Configure rviz

  - Add map and subscribe to the /map topic to avoid error

- Add lasescan and subscribe to /scan topic.

- Move around the world:

  - ros2 run turtlebot3_teleop teleop_keyboard

- Save map

  - ros2 run nav2_map_server map_saver_cli -f ROS_Resources/maps/my_custom_robot_world

- Save rviz configuration in order to not have to recreate step 5 and 6

  - In Rviz2 go to save config as

Once map is saved we can start navigation:

10) ros2 launch nav2_bringup bringup_launch.py use_sim_time:=True map:=ROS_Resources/maps/my_custom_robot_world.yaml

11) Start rviz with navigation:

- ros2 launch turtlebot3_navigation2 navigation2.launch.py   use_sim_time:=True   map:=$HOME/ROS_Resources/maps/my_custom_robot_world.yaml